# Experiment No. 2

# Matlab Code:

clear; clc;

N = 1e6; % number of bits or symbols rng(100); % initialize random generator

ip = rand(1, N) > 0.5; % generating 0,1 with equal probability s = 2*ip - 1; % BPSK modulation: 0 -> -1, 1 -> +1

n = (1/sqrt(2)) * (randn(1, N) + 1j*randn(1, N)); % AWGN noise Eb_N0_dB = -3:10; % range of Eb/N0 values

nErr = zeros(size(Eb_N0_dB)); % preallocate error array for ii = 1:length(Eb_N0_dB)

y = s + 10^(-Eb_N0_dB(ii)/20) * n; ipHat = real(y) > 0;

nErr(ii) = sum(ip ~= ipHat); end

simBer = nErr / N; % simulated BER

theoryBer = 0.5 * erfc(sqrt(10.^(Eb_N0_dB/10))); % theoretical BER figure;

semilogy(Eb_N0_dB, theoryBer, 'b.-', 'LineWidth', 1.5); hold on;

semilogy(Eb_N0_dB, simBer, 'mx-', 'LineWidth', 1.5); axis([-3 10 1e-5 0.5]);

grid on;

legend('Theory', 'Simulation'); xlabel('Eb/N0 (dB)'); ylabel('Bit Error Rate (BER)');

title('Bit Error Probability Curve for BPSK');

clear; clc;

N = 1e6; % number of bits Eb_N0_dB = 0:35;

rng(100); % for reproducibility ip = rand(1, N) > 0.5; % 0/1 bits

s = 2*ip - 1; % BPSK: 0 -> -1, 1 -> +1

nErr = zeros(size(Eb_N0_dB)); % preallocate for ii = 1:length(Eb_N0_dB)

h = (1/sqrt(2)) * (randn(1, N) + 1i*randn(1, N));

n = (1/sqrt(2)) * (randn(1, N) + 1i*randn(1, N)); y = h .* s + 10^(-Eb_N0_dB(ii)/20) * n; h(abs(h) < 1e-10) = 1e-10;

yHat = y ./ h;

ipHat = real(yHat) > 0; nErr(ii) = sum(ip ~= ipHat);

end

simBer = nErr / N;

Eb_N0 = 10.^(Eb_N0_dB/10);

theoryBer = 0.5 * (1 - sqrt(Eb_N0 ./ (1 + Eb_N0))); figure;

semilogy(Eb_N0_dB, theoryBer, 'b-', 'LineWidth', 2); hold on; semilogy(Eb_N0_dB, simBer, 'rx-', 'LineWidth', 1.5);

grid on; xlabel('Eb/N0 (dB)');

ylabel('Bit Error Rate');

title('BER for BPSK in Rayleigh Fading Channel'); legend('Theory','Simulation');

axis([0 35 1e-5 1]);